# VIAVI Osprey Firmware Updater

User Guide - Version 1.0

# Introduction

The VIAVI Osprey Firmware Updater is a software tool designed to update the firmware on your Osprey device and its Micro-Transcoder component. This guide will walk you through the process of connecting your device and performing firmware updates.

# System Requirements

Operating System: Windows

USB-C cable for device connection (can use supplied charging cable)

Firmware files provided by VIAVI Solutions

# Important Safety Information

WARNING: Do not disconnect the USB cable or power off the device during the firmware update process. Doing so may result in device malfunction.

# Installing the VIAVI Osprey Firmware Updater GUI

## Installation Steps

1. Locate the installer file: VIAVI_Osprey_Updater_Setup_v1.0.0.exe

2. Run the installer: Double-click the executable file to begin installation.

3. Windows Defender SmartScreen Warning:

When you first run the installer, Windows Defender SmartScreen may display a warning message stating "Windows protected your PC" with the message "Microsoft Defender SmartScreen prevented an unrecognized app from starting."

This is a standard Windows security feature for applications that haven't been widely distributed yet. The software is safe to install.

To proceed with installation:

Click on "More info" (this link appears in the warning dialog)

A new button labeled "Run anyway" will appear at the bottom of the dialog

Click "Run anyway" to continue with the installation

4. Complete the installation: Follow the on-screen prompts to complete the setup process.

5. Launch the application: Once installed, you can launch the VIAVI Osprey Firmware Updater from your Start Menu or desktop shortcut.

## Troubleshooting

If you continue to experience issues running the installer, ensure that:

You have administrator privileges on your computer

Your antivirus software is not blocking the installation

# Updating Osprey Firmware

- Connect your Osprey device to your computer using a USB-C cable

Launch the VIAVI Osprey Firmware Updater application

Wait for the application to detect your device. The connection status will change from "Device: Not connected" (shown in red) to show the device is connected

In the "Osprey Firmware Update" section, click the "Select Firmware File" button

Navigate to the location where your firmware file is stored

Select the file called “Osprey_X.XX.signed.bin” where X.XX is the version you are trying to upload

The selected filename will be displayed next to the button

Click the "Update Osprey Firmware" button to begin the update process

Monitor the progress bar which will show the update status from 0% to 100%

Wait for the update to complete. Do not disconnect or power off the device during this process

A confirmation message will appear when the update is successful

Power cycle the Osprey

To verify the Osprey firmware has successfully been updated, on the Osprey, go to the “System Info” page and verify that the Version number matches the file you uploaded.

Tip: The progress bar provides real-time feedback on the update status. The entire process typically takes 2-5 minutes.

# Updating Micro-Transcoder

- Connect your Osprey device to your computer using a USB-C cable

Launch the VIAVI Osprey Firmware Updater application

Wait for the application to detect your device. The connection status will change from "Device: Not connected" (shown in red) to show the device is connected

In the "Micro-Transcoder Update" section, click the "Select Hex File" button

Navigate to the location where your hex file is stored

Select the appropriate hex file (.hex extension)

The selected filename will be displayed next to the button

Click the "Update Micro-Transcoder" button to begin the update process

Monitor the progress bar which will show the update status from 0% to 100%

Wait for the update to complete. Do not disconnect the device during this process

A confirmation message will appear when the update is successful

Power cycle the Osprey

# Troubleshooting

## Device Not Detected

Ensure the USB-C cable is properly connected to both the device and computer

Try using a different USB port on your computer

Verify that the USB-C cable supports data transfer (not just charging)

Restart the application and reconnect the device

Power cycle the Osprey

Check Windows Device Manager to ensure the device is recognized. The device will show up as a COM port

## Update Fails or Progress Stops

Ensure you are using the correct firmware/hex file for your device model

Verify the firmware file is not corrupted by re-downloading it

Close other applications that might be accessing USB devices

If the update fails, disconnect and reconnect the device, then try again

## "No file selected" Error

Click the appropriate "Select" button before attempting to update

Ensure the file you selected has the correct extension (.bin for firmware, .hex for transcoder)

# Best Practices

Always use a reliable USB-C cable that supports data transfer

Ensure your laptop or computer is connected to power during the update

Close unnecessary applications before starting the update

Keep backup copies of working firmware versions

Verify firmware version compatibility before updating

Do not disconnect the device until you see the success confirmation message

© VIAVI Solutions | Firmware Update Tool v1.0